| NGN Management Focus Group |  |
| --- | --- |

# NGN Management Specification Roadmap v2.0

# 1.	Purpose and scope

The goal of the work of the NGN Management Focus Group (NGNMFG) is to "organize and undertake a centralized approach regarding specification of NGN management related to FCAPS interfaces, in particular to support the NGN Release 1". FCAPS interfaces are generally understood to include NE-OS and OS-OS interfaces in support of element, network, service, and business management.

Note that the term “centralized” in the goal statement does not imply that the NGNMFG will create all required specifications in a centralized way. The creation of specifications is distributed over many bodies and the responsibilities of these bodies are fully recognized. “Centralized” means here that the main role of the NGNMFG is acting as a coordinator and as an information broker on work directions, plans and progress in the various bodies. In this role, the NGNMFG has a good overview of the field and it can stimulate bodies in filling gaps and resolving harmonization issues.

This roadmap document provides the result of this work. It lists the specifications which the NGNMFG agreed to offer to ITU-T SG4 as candidates for a suite of management specifications. It identifies also the gaps which have to be filled and candidate bodies to undertake this work.

The current content is relevant for NGN Release 1; information regarding later Releases will be added when needed.

# 2.	Specification architecture

The NGNMFG ToR lists a number of objectives. This list provides a high level architecture for the set of specifications to be generated by the NGNMFG.

The specification architecture comprises High Level Requirements (objective 1), Management Requirements (objective 2), Management Framework, Principle and Architecture (objective 3), Protocol-neutral and Protocol-specific NGN Management Interface Specifications (objective 4).

Figure 1 visualizes the specification architecture.

Figure 1: Specification Architecture

Each of the five rectangles in Figure 1 represents a group of specifications, which are contained in one or more documents.

The High Level Requirements are given from outside the NGNMFG, the most important source being the ITU-T NGN Focus Group (NGN FG). The documents containing the high level requirements are:

- Y.2001, General overview of NGN;

- Y.2011, General principles and general reference model for next generation networks;

- Y.NGN-FRA, NGN Functional Requirements and Architecture [1].

The task of the NGNMFG is to identify candidate documents for the remaining four groups. Candidates will be selected from existing documents, from documents which are under construction, or from planned documents. When no suitable candidate can be identified, the NGNMFG will identify a suitable body and invite it to generate the required specification document(s). Only as a very last resort, the NGNMFG will generate a specification.

The suite of candidate documents will be proposed to ITU-T SG4.

# 3.	Specification categories

The set of NGN Management specifications described in this roadmap is separated in four categories:

1) Specifications for NGN management (section 4.1):

Specifications are listed under this category if they have been written specifically for NGN management and their scope is the NGN as a whole, i.e. they are not related to a specific functional entity in the NGN.

Within this category, specifications are grouped according to their role in the specification architecture in Figure 1.

2) Generic telecommunications management specifications, reused for NGN (section 4.2):

Specifications will be listed under this category, if they have not been written specifically for management of the NGN. The indication "NGN" does not appear anywhere in the titles of these documents, yet they are recognized as applicable for management of the NGN because of their generic applicability to telecommunications management.

Dependent on their main subject areas, specifications within this category are identified as generic or according to their relation with the FCAPS and other functional areas.

3) Specifications for the management of NGN functions (section 4.3):

Specifications will be listed under this category if their scope is the management of one or more specific functional entities in the NGN. This category includes specifications which have been written specifically for the management of NGN functions, and specifications which have been written for the management of non-NGN functional entities which are re-used in the NGN.

Specifications in this category are identified according to their main application area in terms of the functional entities of the NGN as they are defined in Y.NGN-FRA [1], Figure 8. These entities are:

- Service Stratum:

  - Application Functions Support Functions and Service Support Functions

  - Service User Profile Functions

  - Service Control Functions:

    - Other Multimedia Components

    - Streaming Service Component

    - PSTN/ISDN Emulation Service Component

    - IP Multimedia Service Component

- Transport Stratum:

  - Transport User Profile Functions

  - Network Access Attachment Function

  - Resource and Admission Control Functions

  - Access Transport Functions

  - Edge Functions

  - Core Transport Functions

- CPE:

  - Legacy Terminals

  - Gateways

  - NGN Terminals

  - Customer Networks

4) Other relevant specifications (section 4.4):

Specifications, considered relevant for the management of the NGN, which do not belong to another category.

# 4.	Agreed specifications

The following tables list the specifications as agreed by the NGNMFG.

## 4.1	Specifications for NGN management

| Subcategory | Document ID | Title | Notes/Issues |
| --- | --- | --- | --- |
| Requirements | ETSI TS 188 003 | OSS Requirements | requirements specifications should be checked for possible harmonisation issues |
|  | ETSI TS 188 004 | NGN Management; OSS Vision |  |
|  | IETF RFC 3535, section 3 | Overview of the 2002 IAB Network Management Workshop |  |
|  | ITU-T M.3060, clause 7 | Management of Next Generation Networks |  |
| Management Architecture | ITU-T M.3060 | Management of Next Generation Networks | harmonization issue with ETSI TS 188 001 (see section 6.1) |
|  | ETSI TS 188 001 | NGN Management; OSS Architecture | harmonization issue with ITU-T M.3060 (see section 6.1) |

## 4.2	Generic telecommunications management specifications reused for NGN

| Subcategory | Document ID | Title | Notes/Issues |
| --- | --- | --- | --- |
| Generic | ITU-T M.3050 series | Enhanced Telecom Operations Map (eTOM) | 1) Equal to TMF eTOM v4 GB921 2) NGN relationship explained in M.3060 and in TS 188 001 |
|  | TMF 053B, C, D, and F | NGOSS Technology-Neutral Architecture v4.5 |  |
|  | TMF GB922 | Shared Information/Data Model (SID) | harmonization issue with CIM, X.alarm-neutral, and MTNM |
|  | DMTF CIM | Common Information Model | harmonization issue with SID |
| Fault | X.alarm | Protocol-Neutral UML Description of the Alarm Reporting Function | harmonisation issue with SID, MTNM, and 32.111 |
| Configuration |  |  |  |
| Accounting | ATIS-0300075-2005 and ATIS-0300075.1-2005 | Usage Data Management for Packet-Based Services - Service-Neutral Architecture and Protocol Requirements - Service-Neutral Protocol Specification for Billing Applications | harmonization issue with 3GPP IMS Charging specifications (see section 6.3) |
| Performance |  |  |  |
| Security | M.3016 series | Security of the management plane | NGN relationship explained in M.3060 |
| Service Management | ITU-T M.3341 | QoS/SLA management service requirements |  |
|  | ITU-T M.3350 | Emergency Telecommunication Service (ETS) management service requirements |  |
| Protocol | IETF Netconf | NETCONF configuration protocol | for interfaces to the NE, for configuration |
|  | IETF STD 62 | Simple Network Management Protocol version 3 | for interfaces to the NE, primarily monitoring (including event reporting) |
|  | OASIS WSDM MUWS | WSDM Management Using Web Services (WSDM-MUWS) v1.1 | for non-NE interfaces |
|  | DMTF WBEM | Web-Based Enterprise Management | for CIM only |
| Interface definition language | IETF STD 58 | Structure of Management Information version 2 | to support SNMP; recommended MIBs are for further study |

Editor’s note: consider adding SG4 M.ngn-ta for trouble ticketing

## 4.3	Specifications for the management of NGN functions

| Subcategory | Document ID | Title | Notes/Issues |
| --- | --- | --- | --- |
| Core Transport Functions | ITU-T G.7718/Y.1709 | Framework for ASON Management |  |
|  | ITU-T G.7718.1/Y.1709.1 | Protocol-neutral management information model for the control plane view |  |
|  | TMF 513 | MTNM Business Agreement (part of TMF MTNM Solution Suite) | harmonisation issues with: - SID, X.alarm-neutral, and 32.111 (see section 6.2) - SID and 32.671-675 (see section 6.4) - MEF 7 and Q.840.1 (see section 6.7) |
|  | TMF 608 | MTNM Information Agreement (part of TMF MTNM Solution Suite) |  |
|  | TMF 814 | MTNM Solution Set in CORBA IDL with Supporting Documentation (part of TMF MTNM Solution Suite) |  |
|  | TMF 814A | MTNM Implementation Statement Templates for CORBA (part of TMF MTNM Solution Suite) |  |
|  | Q.840.1 | Requirements and Analysis for NMS-EMS Management Interface of Ethernet over Transport and Metro Ethernet Network (EoT/MEN) | harmonization issue with MEF 7 and MTNM Solution Suite (see section 6.7) |
|  | MEF 7 | EMS-NMS Information Model (for Metro Ethernet Network) | harmonization issue with Q.840.1 and MTNM Solution Suite (see section 6.7) |
| Access Transport Functions | TMF MTNM Solutions Suite (see Core Transport Functions) |  |  |
|  | Q.838.1 | Requirements and analysis for the management interface of Ethernet passive optical networks (EPON) |  |
| Service Stratum | ATIS TMOC Usage Data Management specifications (see table 4.2) |  |  |
| IMS | 3GPP TS 32.111-1 to -4 | Fault Management IRP | harmonization issue with TMF MTNM Solution Suite, SID, and X.alarm-neutral (see section 6.2) |
|  | 3GPP TS 32.301-304 | Notification IRP |  |
|  | 3GPP TS 32.311-314 | Generic IRP |  |
|  | 3GPP TS 32.601-604 | Basic CM IRP |  |
|  | 3GPP TS 32.621-625 | Generic NRM IRP |  |
|  | 3GPP TS 32.631-635 | Core NRM IRP |  |
|  | 3GPP TS 32.661-664 | Kernel CM IRP |  |
|  | 3GPP TS 32.671- 675 | State Management IRP | possible harmonization issue with TMF MTNM Solution Suite and SID (see section 6.4) |
|  | 3GPP TS 23.125, 32.240, 32.260, 32.296-299 | IMS Charging Management | harmonization issue with TMOC Usage Data Management specifications (see section 6.3) |

Editor’s note: harmonization issues regarding the equipment model need to be investigated

Editor’s note:  3GPP 32 series specifications for the management of UMTS packet switching in Core Transport and UTRAN in Access Transport need to be investigated

## 4.4	Other relevant specifications

| Subcategory | Document ID | Title | Notes/Issues |
| --- | --- | --- | --- |
|  | IETF RFC 3444 | On the Difference between Information Models and Data Models |  |

# 5.	Gaps

The following table identifies areas for which suitable specifications candidate could not be identified. The NGNMFG has brought the gap areas to the attention of the organizations listed in the third column.

| Area | Reference | Organization | Follow up |
| --- | --- | --- | --- |
| Exchange of service orders, trouble tickets, etc | NGNMFG-ID-028 NGNMFG-OD-006 | ITU-T Q.7/4 | working on M.ngn-ta for trouble ticketing |
| NGN function oriented NE MIBs | NGNMFG-ID-135 | ETSI TISPAN WG8 | working on the MIM |

# 6.	Harmonization Issues

## 6.1 	Management Architecture

ETSI TS 188 001 describes the ETSI TISPAN NGN Management Architecture. This architecture is based on Service Oriented Architecture principles. The document specifies the architecture, defining and using architectural concepts like NGN OSS Service, NGN OSS Service Interface, NGN OSS Service Interface Consumer, NGN OSS Operation, and NGN OSS Service Interface. It provides a mapping of the NGN OSS Functional/Information View, which is based on TMForum’s eTOM, to Service Interface Groups.

ITU-T Recommendation M.3060 is titled Principles for the Management of Next Generation Networks. The architecture specified by this document is based on a number of concepts from the TMN Logical Layered Architecture. It refers also to Service Oriented Architecture principles.

#### More investigation is needed to show whether and how the SOA and the LLA based approaches to NGN Management Architecture can be properly unified.

## 6.2	Alarm Management

Alarm management is likely the most basic NGN management function and involves at a minimum alarm reporting with multiple severity levels; it may also include other functionality, such as alarm message routing and alarm message subscription. The following specifications include alarm management capabilities that have been identified for harmonization: TMF MTNM Solution Suite, TMF SID, 3GPP SA5 32.111, and ITU-T SG 4 X.alarm-neutral.

## 6.3	Accounting, Charging, Billing

3GPP SA5 and ATIS TMOC have reached an initial agreement regarding the harmonization of their specifications for charging, billing, and accounting management and have indicated their willingness to work with the NGNMFG and SG4 to "move this work forward." This agreement provides guidelines on how to utilize the work of both organizations to support the architecture and functional requirements in Y.NGN-FRA.

## 6.4	State Management

State management is another basic NGN management function. The following specifications include state management capabilities that have been identified for harmonization: TMF MTNM Solution Suite, TMF SID, and 3GPP SA5 32.671-675.

## 6.5	Generic Information Models

There are two Generic Information Models which were both inputs to the NGN Roadmap - TMF Shared Information and Data Model (SID) and DMTF Common Information Model (CIM). Traditionally the CIM and SID models were focusing on different operational domains. Currently, CIM’s primary focus is on the IT resource management (Systems, Storage) and IP Networks, and SID provides a description of the telecom service provider's total technical and business systems. However, both models have a lot of similarities and overlap in the domain they cover and are currently used by the number of different management solutions.

Harmonization between CIM and SID needs to occur within operational domains where these models need to coexist in order to provide interoperability between different management solutions used for NGN.

The CIM-SID Harmonization activity is a joint effort between the Distributed Management Task Force (DMTF) and the TeleManagement Forum (TMF). The project is divided into two phases. Phase 1 resulted in the publication of two companion documents: “DMTF/TMF Model Alignment Physical sub-Model Alignment”, DMTF DSP2004/TMF GB932, and “DMTF/TMF Model Alignment SID Logical Resources and CIM Networks Sub-Models”, DMTF DSP2000/TMF GB933. The goal of Phase 2 is to identify processes and guidelines for the CIM-SID model mapping. This effort is scoped to the Physical and Logical Resources as identified in Phase 1. The final objective, however, is to develop an approach applicable to other domains as well.

Further detail on this work is contained within Annex 4.

## 6.6	Model Harmonization

An initial face-to-face meeting began with the presentation of the model status and planning in the following organizations: ETSI TISPAN WG8, TeleManagement Forum, 3GPP SA5, Distributed Management Task Force, and ITU-T SG 4.

With the above background information, the meeting focused on how to achieve a "shared/common model" and led to a set of goals, status, agreements, and issues and a plan for going forward. The centerpiece of the meeting's agreements was a proposal for a Shared Information Architecture (SIA) consisting of 3 model tiers: Tier 1 provides an overarching protocol-neutral model which "hosts" or maps onto a set of Tier 2 protocol-neutral models focused on specific problem domains which in turn map onto a set of Tier 3 implementable models based on specific paradigms.

## 6.7	Ethernet Harmonization

802.3-based ethernet technology is a key component of the NGN transport stratum. The following specifications include ethernet management capabilities that have been identified for harmonization: ITU-T SG 4 Q.840.1, MEF 7, and TMF MTNM Solution Suite.

______________

Annex 1: Terms of reference for the NGNMFG

Title:	Telecom Management Collaboration Focus Group for NGN Management

Chair:			Dave Sidor

Vice Chair:		Leen Mak

1.	Introduction

The NGN Management Focus Group (NGNMFG) was established in response to a request to the SG 4 Management Team from the initial meeting of the Focus Group on NGN (FGNGN) to organize and undertake a centralized approach regarding specification of NGN management related to FCAPS interfaces, in particular to support NGN Release 1. Under the provisions of ITU-T Recommendation A.7, and with the agreement of the SG 4 Management Team, the ITU-T Director, and the TSAG Chairman, the NGNMFG was organized under the sponsorship of SG 4. The NGNMFG establishment was confirmed by SG 4 in February 2005 and re-confirmed in September 2005. As stated in this document, the NGNMFG is expected to complete its activities in May 2006 and provide a summary of its activities in a report to the SG 4.

Note: By FCAPS is meant Fault, Configuration, Accounting, Performance, and Security Management and within the ITU-T the phrase "FCAPS interfaces" generally includes NE-OS and OS-OS interfaces in support of element, network, service, and business management. However it should be noted that outside the ITU-T the acronym FCAPS has a varied connotation and thus in this document the phrase "management interfaces" will be used to describe these interfaces.

2.	Objectives

The general objectives are to provide the management interface capabilities to support the ITU-T (including FGNGN) objectives for Release 1 and more specific objectives are presented below:

1) To understand and agree with the high level requirements related or relevant to management interfaces that are contained in the NGN Overview (Y.2001), NGN General Reference Model (Y.2011), and the NGN Functional Requirements and Architecture (Y.NGN-FRA) and to determine from them, the functional and physical entities to be managed for Release 1.

2) To maintain and document a liaison relationship with supporting organizations, including those identified in clause 3.

3) To enhance the format and contents of the NGN Management Specification Roadmap (v1.0) for Release1 (see SG 4 TD 144 Add1 GEN) that identifies the following categories of specifications relevant to NGN Management:

- Management requirements

- Management framework, principles, and architecture

- Management interface specifications, both protocol-neutral and protocol-specific

To achieve this objective, the NGNMFG will identify existing and in progress specifications available to support the management needs of Release 1, in particular by examining the output of the organizations noted in clause 3 below.

4) In addition, to identify and coordinate with the relevant organizations

- The generation of additional specifications needed to complete the Roadmap for Release 1 ("filling the gaps")

- The harmonization of "overlapping" specifications

5) To recommend, if appropriate, to non-ITU organizations the submission of relevant specifications for consideration as ITU-T Recommendations.

6) As a last resort, to produce specifications deemed to be necessary but unavailable from any source.

7) To provide a high level plan for additional NGN releases.

8) To document the Focus Group decisions, including a revised NGN Management Specification Roadmap and any other recommendations, regarding clauses 1-7 above for submission to SG 4 and the other participating organizations.

9)  Produce a working model to continue the development and execution of the NGN Management work plan after the NGNMFG is terminated, including the identification of areas requiring additional work or harmonization. This model should continue the successful multi-SDO/forum working process that has been nurtured by the NGNMFG.

3. 	Participation

Participation is open to individuals from organizations with expertise and specifications applicable to NGN management interfaces, including the following:

- ITU-T SGs 4 and 15

- ETSI TISPAN (WG8)

- ATIS TMOC

- TTC

- 3GPP SA5

- 3GPPS TSG-S (WG5)

- TeleManagement Forum

- IETF Operations and Management Area

- OASIS (WSDM)

- OSS/J

- MEF

- DMTF

- IPDR

4.	Time schedule

The general objective of the NGNMFG is to provide the management interface capabilities to support the ITU-T (including FGNGN) objectives for Release 1. Therefore the time schedule for the completion of the work of the NGNMFG takes into account the currently unknown timeframe for the completion of the NGN Release 1 documentation. Based on available information, the following schedule is planned:

- 2Q2006: NGNMFG generates report including Roadmap v2.0, for consideration by SG4 (copy to SG 13). Roadmap v2.0 will support NGN Release 1 as far as it has been approved in the January 2006 meeting of SG13.

If the NGN Release 1 specification work has not been completed in the January 2006 meeting of SG13, the May 2004 meeting of SG4 will be asked to recharter the NGNMFG, with the following additional task:

- 1Q2007: NGNMFG generates report including Roadmap v3.0, finalizing Release 1, for consideration by SG 4 (copy to SG 13).

Together with its final report (either in 2Q2006 or 1Q2007), NGNMFG generates a plan for the continuation of the development and execution of the NGN Management work plan after the NGNMFG is terminated.

5.	Method of Working

Registration is required to participate in the activities of the NGNMFG or to access its documents. For registration and other information, see the NGNMFG web site at http://www.itu.int/ITU-T/studygroups/com04/ngn-mfg/index.html

It is expected that the Focus Group will make its decisions by consensus among the participants at the meeting.

If necessary, more detailed working procedures may be produced by the Focus Group.

Use of electronic means, including email, teleconferences, collaborative software, etc. is preferred. However face-to-face meetings are also possible.

A mailing list dedicated to this activity (ngnmfg@iut.int) has been established. In any case any general announcements and other general information regarding the Focus Group will also be disseminated via the SG 4 general mailing list.

The Focus Group may generate liaisons to the participating organizations as needed.

Any specification created as a result of the clause 2, item 5, should preferably use the template of an ITU-T Recommendation.

6.	Focus Group Document Submission Process

Members submit document(s) by email to the NGNMFG mailing list and/or upload to the NGNMFG ftp area (refer to the most recent version of document NGNMFG-AD-001 for further details).

Material such as text, diagrams, etc., submitted as input to the work of this Focus Group is presumed by the Focus Group to have no restrictions in order to permit the normal distribution of this material for discussions within the Focus Group and possible use, in whole or in part, in any resulting Focus Group document submitted to SG 4 for publication. By submitting input to the Focus Group, authors acknowledge this condition of submission. In addition, authors may state any specific conditions on other uses of their contribution.

7.	Document Usage Policy

Usage of Focus Group documents, published on the web/ftp site, will be governed by the following policy:

"The documents on this web/ftp site are provided to further the aims of the Focus Group only. Participants are not allowed to use these documents for purposes that go beyond the scope of the Focus Group work, without prejudice to other rights a participant may have with respect to any of these documents. If any specification contributed to the NGNMFG is shared with others not participating in the NGNMFG, the sharer is requested to make the recipients aware of the above constraint."

8.	Finances

As noted above, the Focus Group will primarily work electronically. However face-to-face meetings hosted by one or more of the participants may be planned as well.

History

| Issue 0.1 | 1 September 2004 |  |
| --- | --- | --- |
| Issue 0.2.2 | 21 September 2004 |  |
| Issue 1.1 | 22 September 2004 |  |
| Issue 1.2 | 12 January 2005 |  |
| Issue 1.3 | 4 February 2005 |  |
| Issue 2.1 | 25 February 2005 | updated clause 4 to add interim report and indicate firm dates, replaced first sentence in clause 5 with registration text and URL, and modified this table toadd history rationale |
| Issue 3.0 | 27 September 2005 | Proposed revision for continuing the NGNMFG, emphasizing the objective of enhancing of the NGN Management Specification Roadmap by filling gaps and harmonizing overlaps |
| Issue 3.1 | 28 September 2005 | Revised time schedule and added plan for post-NGNMFG work approach |
| Issue 3.2 | 16 May 2006 | Removed NE/OS-WS interfaces from Introduction |

_______________

### Annex 2: Organizations owning the listed specifications

In this document, abbreviated names are used to identify the organizations which own the contributed specifications. This appendix lists the abbreviations, the full names, and the urls of the websites of the contributing organizations.

| Acronym | Full name | Website |
| --- | --- | --- |
| 3GPP | 3rd Generation Partnership Project | www.3gpp.org |
| ATIS | Alliance for Telecommunications Industry Solutions | www.atis.org |
| DMTF | Distributed Management Task Force | www.dmtf.org |
| ETSI | European Telecommunications Standards Institute | www.etsi.org |
| IETF | Internet Engineering Task Force | www.ietf.org |
| ITU-T | International Telecommunication Union | www.itu.int/itu-t |
| MEF | #### Metro Ethernet Forum | www.metroethernetforum.org |
| OASIS | #### Organization for the Advancement of Structured Information Standards | www.oasis-open.org |
| TMF | TeleManagement Forum | www.tmforum.org |

_______________

### Annex 3: Description of Managed Entities

The following documents contain specifications and descriptions of the NGN Managed Entities.

| ##### Identification | Title | Notes |
| --- | --- | --- |
| 3GPP 23.002 | Network architecture |  |
| 3GPP 23.228 | IP Multimedia Subsystem (IMS); Stage 2 |  |
| ITU-T Y.NGN-FRA | Functional Requirements and Architecture of the NGN |  |
| ITU-T Y.INF | IMS for Next Generation Networks |  |
| ETSI ES 282 001 | NGN Functional Architecture Release 1 |  |
| ETSI ES 282 002 | PSTN/ISDN Emulation Sub-system (PES); Functional architecture |  |

Editor’s note: other specifications to be added

_______________

### Annex 4: CIM-SID Harmonization activity – Progress Report (May 2006)

The CIM-SID Harmonization activity is a joint effort between the Distributed Management Task Force (DMTF) and the TeleManagement Forum (TMF). The working group started more than a year and a half ago. This project is divided into two phases; an overview is provided below.

##### CIM-SID Harmonization, Phase 1 Overview

The project is divided into two phases.  Phase 1 resulted in the publication of two companion documents: “DMTF/TMF Model Alignment Physical sub-Model Alignment”, DMTF DSP2004/TMF GB932, and “DMTF/TMF Model Alignment SID Logical Resources and CIM Networks Sub-Models”, DMTF DSP2000/TMF GB933. This work identifies the scope of the models, similarities and differences with respect to the modeling approach, major concepts captured in the models, and related classes.

GB932 (Physical Resources) provides an evaluation and high level comparison between GB922 Addendum 5PR and selected areas of the DMTF CIM 2.10 Physical Model.

GB933 (Logical Resources) provides an evaluation and high level comparison, within the domain of Communication Networks, between selected areas of GB922 Addendum 5LR – Logical Resource Business Entity Definitions (v1.1) defined as part of the Shared Information/Data (SID) Model and the Networks Model subset of CIM v2.10.

As noted, GB932 and GB933 identify the key concepts appropriate to their respective domains and describe how they are represented in each of the models. The conclusion in GB932 indicates that there is a great deal of similarity between the CIM and the SID Physical Models and mapping is possible. Nevertheless, in order to enable the automation of this process some changes could be made. Details of the possible changes are left for Phase 2. GB933 concludes by stating that simple federation between the two models is not feasible due to major differences in design approach and scope. However, it may be possible to identify a correspondence between common concepts between the CIM and SID. In order to accomplish this, it is likely that modifications will be needed to one or both models to make subsequent mappings based on these correspondences more straightforward.

##### CIM-SID Harmonization, Phase 2 Overview

The goal of Phase 2 is to identify processes and guidelines for the CIM-SID model mapping. This effort is scoped to the Physical and Logical Resources as identified in Phase 1.  The final objective, however, is to develop an approach applicable to other domains as well.  The group is proceeding with an in-depth analysis based on:

- Defining scenarios/use cases relevant to CIM- and SID-centric operational domains.

- Deriving a set of semantic concepts from the use cases.

- Organizing the semantic concepts into a higher-level ontology that captures their interactions.

- Mapping CIM and  SID elements onto the identified semantic concepts.

- Inferring CIM-SID element mappings.

Example deployment scenario and business use case:

The following deployment scenario and high-level business use case is proposed as an initial representative of an important domain where CIM and SID may need to co-exist.

Customer: A traditional telecommunications service provider (SP) is rolling out IP Multimedia Subsystem (IMS).

Architecture: On one side the SP has the IP Multimedia Subsystem (IMS) core and radio access network and on the other side an IT infrastructure that supports the vast majority of the value added services being made available to subscribers/end-users.

Assumptions:

The IMS core and radio access network expose resource management interfaces through the Integration Reference Points (IRPs) defined by 3GPP.  The corresponding Network Resource Models (NRMs) are realized on specified protocols such as Simple Network Management Protocol (SNMP), Web Based Enterprise Management (WBEM), or Common Object Resource Broker Architecture (CORBA).

The IP networking environment underlying both the IMS core and the IT infrastructure exposes CIM-based resource management interfaces.

The SP's Operations Support System (OSS) uses Operations Support Systems through Java (OSS/J) APIs for Order Management, Service Quality Management, Trouble Ticketing, Billing Mediation and Quality of Service (QoS).

Requirement: The Order Management System must be used for order fulfillment in the IMS environment.

Conclusion: In order to meet the requirement for reuse of the SP's existing OSS/J implementation, CIM-based representations of the IMS NRMs and IP infrastructure resources must be mapped to corresponding elements of the OSS/J Core Business Entities (CBE) (and therefore of the SID).

Model Comparison and Mapping Methodology:

In general it is possible to identify a set of semantic concepts shared between different models within the scope of particular business use cases and deployment scenarios; each model would represent these concepts differently.

The set of concepts that collectively abstracts a domain can itself be described as a higher-level ontology.  An example in the domain of Communication Networks is Topology, Network, SubNetwork, Network Elements, Connectivity,  Protocols, etc. Element-level mappings can be made across the different models by mapping these concepts into the respective model elements.

Semantic concepts may be mapped to the SID via the patterns defined by the Business View Entities (TMF GB922).  The semantic concepts may be mapped to the CIM by leveraging CIM Profiles.

Mapping of the semantic concepts to a particular model is done in the context of business processes expressed as high-level Use Cases.

The set of Use Cases scoped to a particular domain (e.g. “Communication Networks”) implies key concepts that must be represented.

The initial goal of the mapping effort is to represent each of the semantic concepts for a given problem space using both CIM and SID.  Initial efforts will concentrate on structural/relational mappings.  Behavioral analysis will be undertaken as needed to support the semantic and structural mappings.

Phase 2 Deliverables:

The Deliverables for Phase 2 will include:

- Definition of deployment scenarios within an operational domain where CIM and SID need to coexist.

- A set of semantic concepts derived from these use cases.

- A mapping methodology.

- A mapping between the relevant CIM and SID model elements by applying the proposed  mapping methodology to the concrete examples of Equipment and Network modeling.

_______________

### Annex 5: Information about the listed specifications

Annex 5 contains bibliographic information for each of the specifications listed in this document. This appendix is contained in a separate powerpoint file.

_______________

### References

[1]	Y.NGN-FRA, Functional Requirements and Architecture of the NGN Version 0.1 (ITU-T TD 133 (WP 2/13))

______________

### Document history

| Version | Published in | Comments |
| --- | --- | --- |
| 0.0 | NGNMFG-ID-045 | input to 31 March 2005 meeting |
| 0.1 | NGNMFG-ID-045-R1 | output of 31 March 2005 meeting |
| 0.2 | NGNMFG-ID-045-R2 | output of 26 May 2005 meeting |
| 0.3 | NGNMFG-ID-045-R3 | output of 22 June 2005 meeting |
| 0.4 | NGNMFG-ID-045-R4 | output of 27 July 2005 meeting |
| 0.5 | NGNMFG-OD-010-R2 | output of 28 July 2005 meeting |
| 1.0 | NGNMFG-OD-013-R2 | output of 7 September 2005 meeting (doc id: NGNMFG-OD-013-R2_Final_Roadmap.doc) |
| 1.1 | Permanent Document | header format adapted input to 1 November 2005 meeting |
| 1.2 | Permanent Document | agreed output of January, February, March, April 2006 meetings |
| 2.0 | Permanent Document | changes made after April meeting, before SG4 meeting, reviewed per email; reported to SG4 in NGNMFG-OD-019 |

_______________

NGNMFG Terms of Reference, section 1 [Annex 1]