| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 690 Rev.1 (PLEN/15) |
|  |  | English only Original: English |
| Question(s): | 14/15 | 10-21 September 2012 |
| TD |  |  |
| Source: | Editor G.8151/Y.1374 |  |
| Title: | Draft Amendment 1 to Recommendation ITU-T G.8151/Y.1374 (2012) (for Consent, September 2012) |  |

This document contains for consent the draft Amendment 1 of G.8151/Y.1374 “Management aspects of the MPLS-TP network element”.

This Amendment 1 contains the updates to G.8151/Y.1374 to align with the MPLS-TP equipment functional blocks updates described in G.8121 Amendment 1.

Draft Amendment 1 to Recommendation ITU-T G.8151/Y.1374 (2012)

Management aspects of the MPLS-TP network element

Amendment 1

Summary

This amendment contains updates to G.8151/Y.1374 (07/2012) to align with the MPLS-TP equipment function blocks updates described in G.8121 Amendment 1.

Draft Amendment 1 to Recommendation ITU-T G.8151/Y.1374 (2012)

Management aspects of the MPLS-TP network element

Amendment 1

# 1)	Scope

This amendment contains updates to G.8151/Y.1374 (07/2012) to align with the MPLS-TP equipment function blocks updates described in G.8121 Amendment 1.

# 2)	References

- G.8151/Y.1374 (76/12) Management aspects of the MPLS-TP network element

# 3)	Updates to G.8151

# 3.1)	Updates to clause 8.5 “Adaptation”

# Modify Table 8-2 as highlighted in the following replacement table for the deletions and additions.

# Table 8-2/G.8151/Y.1374  Provisioning and reporting for adaptation functions

| MI Signal | Value Range | Default Value |
| --- | --- | --- |
| MT/MT_A_So Provisioning |  |  |
| MT/MT_A_So_MI_Admin_State | LCK, Normal | Normal |
| MT/MT_A_So_MI_Label [1…M] | 16 to (2**20)-1 | Note-2 |
| MT/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | Note-1 |
| MT/MT_A_So_MI_CoS[1…M] | Note-1 | Note-1 |
| MT/MT_A_So_MI_PHB2EXPMapping[1…M] | Note-1 | Note-1 |
| MT/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-2 |
| MT/MT_A_So_MI_LCK_Period[1…M] | 1 s, 1 min | 1 s |
| MT/MT_A_So_MI_LCK_CoS[1…M] | 0..7 | 7 |
| MT/MT_A_So_MI_GAL_Enable[1...M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MT/MT_A_Sk Provisioning |  |  |
| MT/MT_A_Sk_MI_Admin_State | LCK, Normal | Normal |
| MT/MT_A_Sk_MI_Label [1…M] | 16 to (2**20)-1 | Note-2 |
| MT/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | Note-1 |
| MT/MT_A_Sk_MI_CoS[1…M] | Note-1 | Note-1 |
| MT/MT_A_Sk_MI_ TC2PHBMapping[1…M] | Note-1 | Note-1 |
| MT/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-2 |
| MT/MT_A_Sk_MI_AIS_Period[1...M] | 1 s, 1 min | 1 s |
| MT/MT_A_Sk_MI_AIS_CoS[1...M] | 0..7 | 7 |
| MT/MT_A_Sk_MI_LCK_Period[1...M] | 1 s, 1 min | 1 s |
| MT/MT_A_Sk_MI_LCK_CoS[1...M] | 0..7 | 7 |
| MT/MT_A_Sk_MI_GAL_Enable [1...M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MTDi/MT_A_Sk Provisioning |  |  |
| MTDi/MT_A_Sk_MI_DS_MP_Type Note: This MI should be properly configured by the EMF on the basis of the MPLS-TP connection configuration within the node but not exposed to the operator as a configuration parameter in the NE/EMS management interface. See G.8121 clause 9.4.2.2.2 and its Appendix I for examples of configuration of this MI. | MEP, MIP | -- |
| MT/ETH_A_So Provisioning |  |  |
| MT/ETH_A_So_MI_Admin_State | LCK, Normal | Normal |
| MT/ETH_A_So_MI_FCSEnable | true, false | true |
| MT/ETH_A_So_MI_CWEnable | true, false | true |
| MT/ETH_A_So_MI_SQUse | true, false | false |
| MT/ETH_A_So_MI_PRI2CoSMapping | Note-1 | Note-1 |
| MT/ETH_A_So_MI_MEP_MAC* | 6 byte Unicast MAC address | -- |
| MT/ETH_A_So_MI_Client_MEL* | 0..7 | 7 |
| MT/ETH_A_So_MI_LCK_Period* | 1 s, 1 min | 1 s |
| MT/ETH_A_So_MI_LCK_Pri* | 0..7 | 7 |
| MT/ETH_A_So_MI_MEL* | 0..7 | 7 |
| MT/ETH_A_Sk Provisioning |  |  |
| MT/ETH_A_Sk_MI_FCSEnable | true, false | true |
| MT/ETH_A_Sk_MI_CWEnable | true, false | false |
| MT/ETH_A_Sk_MI_SQUse | true, false | false |
| MT/ETH_A_Sk_MI_CoS2PRIMapping | Note-1 | Note-1 |
| MT/ETH_A_Sk_MI_MEL* (Note: * ETH OAM related) | 0..7 | 7 |
| MT/ETH_A_Sk_MI_Admin_State | LCK, Normal | Normal |
| MT/ETH_A_Sk_MI_LCK_Period * | 1 s, 1 min | 1 s |
| MT/ETH_A_Sk_MI_LCK_Pri * | 0..7 | 7 |
| MT/ETH_A_Sk_MI_Client_MEL * | 0..7 | 7 |
| MT/ETH_A_Sk_MI_MEP_MAC * | 6 byte Unicast MAC address | -- |
| MT/ETH_A_Sk_MI_AIS_Pri * | 0..7 | 7 |
| MT/ETH_A_Sk_MI_AIS_Period * | 1 s, 1 min | 1 s |
| MT/SCC_A_So Provisioning |  |  |
| MT/SCC_A_So_MI_Active | true, false | true |
| MT/SCC_A_So_MI_ECC_CoS | 0..7 | 7 |
| MT/SCC_A_So_MI_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MT/SCC_A_Sk Provisioning |  |  |
| MT/SCC_A_Sk_MI_Active | true, false | true |
| MT/SCC_A_Sk_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MT/MCC_A_So Provisioning |  |  |
| MT/MCC_A_So_MI_Active | true, false | true |
| MT/MCC_A_So_MI_ECC_CoS | 0..7 | 7 |
| MT/MCC_A_So_MI_GAL_enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MT/MCC_A_Sk Provisioning |  |  |
| MT/MCC_A_Sk_MI_Active | true, false | true |
| MT/MCC_A_Sk_MI_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Sn/MT_A_So Provisioning |  |  |
| Sn/MT_A_So_MI_SCCType | 0..255 | 32 |
| Sn/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sn/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Sn/MT_A_So_MI_CoS[1…M] | 0..7 | -- |
| Sn/MT_A_So_PHB2TCMapping[1…M] | Note-1 | -- |
| Sn/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | -- |
| Sn/MT_A_Sk Provisioning |  |  |
| Sn/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| Sn/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sn/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | Note-1 |
| Sn/MT_A_Sk_MI_CoS[1…M] | Note-1 | Note-1 |
| Sn/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| Sn/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| Sn/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| Sn/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| Sn/MT_A_Sk_MI _LCK_OAM_Tool [1…M] | G.8113.1, G.8113.2 | N/A |
| Sn/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| Sn/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| Sn/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| Sn/MT_A_Sk_MI _AIS_OAM_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| Sn/MT_A_Sk_MI _GAL_enable[1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Sn/MT_A_Sk Reporting |  |  |
| Sn/MT_A_Sk_MI_AcSL (see Table 9-11 of G.707) | 0..255 | -- |
| Sn/MT_A_Sk_MI_AcEXI (see Table 6-2 of G.7041) | 0..15 | -- |
| Sn/MT_A_Sk_MI_LastValidUPI (see Table 6-3 of G.7041) | 0..255 | -- |
| Sn-X-L/MT_A_So Provisioning |  |  |
| Sn-X-L/MT_A_So_MI_SCCType (See Table 6-3 of G.7041) | 0..255 | 32 |
| Sn-X-L/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sn-X-L/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Sn-X-L/MT_A_So_MI_CoS[1…M] | 0..7 | Note-1 |
| Sn-X-L/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| Sn-X-L/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| Sn-X-L/MT_A_Sk Provisioning |  |  |
| Sn-X-L/MT_A_Sk_MI_SCCType (See Table 6-3 of G.7041) | 0..255 | 32 |
| Sn-X-L/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sn-X-L/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Sn-X-L/MT_A_Sk_MI_CoS[1…M] | 0..7 | Note-1 |
| Sn-X-L/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| Sn-X-L/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| Sn-X-L/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| Sn-X-L/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| Sn-X-L/MT_A_Sk_MI _LCK_OAM_Tool [1…M] | G.8113.1, G.8113.2 | N/A |
| Sn-X-L/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| Sn-X-L/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| Sn-X-L/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| Sn-X-L/MT_A_Sk_MI _AIS_OAM_Tool [1…M] | G.8113.1, G.8113.2 | N/A |
| Sn-X-L /MT_A_Sk_MI _GAL_Enable [1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Sn-X-L/MT_A_Sk Reporting |  |  |
| Sn-X-L/MT_A_Sk_MI_AcSL (see Table 9-11 of G.707) | 0..255 | -- |
| Sn-X-L/MT_A_Sk_MI_AcEXI (see Table 6-2 of G.7041) | 0..15 | -- |
| Sn-X-L/MT_A_Sk_MI_LastValidUPI (see Table 6-3 of G.7041) | 0..255 | -- |
| Sm/MT_A_So Provisioning |  |  |
| Sm/MT_A_So_MI_SCCType | 0..255 | 32 |
| Sm/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sm/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Sm/MT_A_So_MI_CoS[1…M] | 0..7 | -- |
| Sm/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| Sm/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| Sm/MT_A_Sk Provisioning |  |  |
| Sm/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| Sm/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sm/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | --- |
| Sm/MT_A_Sk_MI_CoS[1…M] | 0..7 | -- |
| Sm/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| Sm/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| Sm/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| Sm/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| Sm/MT_A_Sk_MI _LCK_OAM_Tool [1…M] | G.8113.1, G.8113.2 | N/A |
| Sm/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| Sm/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| Sm/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| Sm/MT_A_Sk_MI _AIS_OAM_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| Sm/MT_A_Sk_MI _GAL_Enable[1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Sm/MT_A_Sk Reporting |  |  |
| Sm/MT_A_Sk_MI_AcSL (see Table 9-12 and Table 9-13 of G.707) | 0..255 | -- |
| Sm/MT_A_Sk_MI_AcEXI (see Table 6-2 of G.7041) | 0..15 | -- |
| Sm/MT_A_Sk_MI_LastValidUPI (see Table 6-3 of G.7041) | 0..255 | -- |
| Sm-X-L/MT_A_So Provisioning |  |  |
| Sm-X-L/MT_A_So_MI_SCCType | 0..255 | 32 |
| Sm-X-L/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sm-X-L/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Sm-X-L/MT_A_So_MI_CoS[1…M] | 0..7 | -- |
| Sm-X-L/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| Sm-X-L/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| Sm-X-L/MT_A_Sk Provisioning |  |  |
| Sm-X-L/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| Sm-X-L/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Sm-X-L/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Sm-X-L/MT_A_Sk_MI_CoS[1…M] | 0..7 | -- |
| Sm-X-L/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| Sm-X-L/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| Sm-X-L/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| Sm-X-L/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| Sm-X-L/MT_A_Sk_MI _LCK_OAM_Tool [1…M] | G.8113.1, G.8113.2 | N/A |
| Sm-X-L/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| Sm-X-L/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| Sn-X-L/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| Sm-X-L/MT_A_Sk_MI _AIS_OAM_Tool [1…M] | G.8113.1, G.8113.2 | N/A |
| Sm-X-L/MT_A_Sk_MI _GAL_Enable[1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Sm-X-L/MT_A_Sk Reporting |  |  |
| Sm-X-L/MT_A_Sk_MI_AcSL | 0..255 | -- |
| Sm-X-L/MT_A_Sk_MI_AcEXI | 0..15 | -- |
| Sm-X-L/MT_A_Sk_MI_LastValidUPI | 0..255 | -- |
| Pq/MT_A_So Provisioning |  |  |
| Pq/MT_A_So_MI_SCCType | 0..255 | 32 |
| Pq/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Pq/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Pq/MT_A_So_MI_CoS[1…M] | 0..7 | -- |
| Pq/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| Pq/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| Pq/MT_A_Sk Provisioning |  |  |
| Pq/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| Pq/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Pq/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Pq/MT_A_Sk_MI_CoS[1…M] | 0..7 | -- |
| Pq/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| Pq/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| Pq/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| Pq/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| Pq/MT _A_Sk_MI _LCK_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| Pq/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| Pq/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| Pq/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| Pq/MT _A_Sk_MI _AIS_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| Pq/MT _A_Sk_MI _GAL_Enable [1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Pq/MT_A_Sk Reporting |  |  |
| Pq/MT_A_Sk_MI_AcSL (see Clause 2.1.2 of G.832) | 0..7 | -- |
| Pq/MT_A_Sk_MI_AcEXI (see Table 6-2 of G.7041) | 0..15 | -- |
| Pq/MT_A_Sk_MI_LastValidUPI (see Table 6-3 of G.7041) | 0..255 | -- |
| Pq-X-L/MT_A_So Provisioning |  |  |
| Pq-X-L/MT_A_So_MI_SCCType | 0..255 | 32 |
| Pq-X-L/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Pq-X-L/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Pq-X-L/MT_A_So_MI_CoS[1…M] | o..7 | -- |
| Pq-X-L/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| Pq-X-L/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| Pq-X-L/MT_A_Sk Provisioning |  |  |
| Pq-X-L/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| Pq-X-L/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| Pq-X-L/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| Pq-X-L/MT_A_Sk_MI_CoS[1…M] | 0..7 | -- |
| Pq-X-L/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| Pq-X-L/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| Pq-X-L/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| Pq-X-L/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| Pq-X-L/MT _A_Sk_MI _LCK_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| Pq-X-L/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| Pq-X-L/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| Pq-X-LMT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| Pq-X-L/MT _A_Sk_MI _AIS_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| Pq-X-L//MT _A_Sk_MI _GAL_Enable[1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| Pq-X-L/MT_A_Sk Reporting |  |  |
| Pq-X-L/MT_A_Sk_MI_AcSL | 0..7 | -- |
| Pq-X-L/MT_A_Sk_MI_AcEXI | 0..15 | -- |
| Pq-X-L/MT_A_Sk_MI_LastValidUPI | 0..255 | -- |
| ODUkP/MT_A_So Provisioning |  |  |
| ODUkP/MT_A_So_MI_Active | true, false | false |
| ODUkP/MT_A_So_MI_SCCType | 0..255 | 32 |
| ODUkP/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| ODUkP/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| ODUkP/MT_A_So_MI_CoS[1…M] | 0..7 | -- |
| ODUkP/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| ODUkP/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| ODUkP/MT_A_Sk Provisioning |  |  |
| ODUkP/MT_A_Sk_MI_Active | true, false | false |
| ODUkP/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| ODUkP/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| ODUkP/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| ODUkP/MT_A_Sk_MI_CoS[1…M] | 0..7 | -- |
| ODUkP/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| ODUkP/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| ODUkP/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| ODUkP/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| ODUkP/MT _A_Sk_MI _LCK_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| ODUkP/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| ODUkP/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| ODUkP/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| ODUkP/MT _A_Sk_MI _AIS_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| ODUkP/MT _A_Sk_MI _GAL_Enable[1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| ODUkP/MT_A_Sk Reporting |  |  |
| ODUkP/MT_A_Sk_MI_AcPT (see Table 15-8 of G.709) | 0..255 | -- |
| ODUkP/MT_A_Sk_MI_AcEXI (see Table 6-2 of G.7041) | 0..15 | -- |
| ODUkP/MT_A_Sk_MI_LastValidUPI (see Table 6-3 of G.7041) | 0..255 | -- |
| ODUkP-X-L/MT_A_So Provisioning |  |  |
| ODUkP-X-L/MT_A_So_MI_Active | true, false | false |
| ODUkP-X-L/MT_A_So_MI_SCCType | 0..255 | 32 |
| ODUkP-X-L/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| ODUkP-X-L/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| ODUkP-X-L/MT_A_So_MI_CoS[1…M] | 0..7 | -- |
| ODUkP-X-L/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| ODUkP-X-L/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-1 |
| ODUkP-X-L/MT_A_Sk Provisioning |  |  |
| ODUkP-X-L/MT_A_Sk_MI_Active | true, false | false |
| ODUkP-X-L/MT_A_Sk_MI_SCCType | 0..255 | 32 |
| ODUkP-X-L/MT_A_Sk_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| ODUkP-X-L/MT_A_Sk_MI_LSPType[1…M] | E-LSP, L-LSP | -- |
| ODUkP-X-L/MT_A_Sk_MI_CoS[1…M] | 0..7 | -- |
| ODUkP-X-L/MT _A_Sk_MI _LCK_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| ODUkP-X-L/MT_A_Sk_MI_TC2PHBMapping[1…M] | Note-1 | Note-1 |
| ODUkP-X-L/MT_A_Sk_MI_QoSDecodingMode[1…M] | A, B | Note-1 |
| ODUkP-X-L/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| ODUkP-X-L/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | -- |
| ODUkP-X-L/MT _A_Sk_MI _AIS_Tool[1…M] | G.8113.1, G.8113.2 | N/A |
| ODUkP-X-L/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| ODUkP-X-L/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| ODUkP-X-L/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | -- |
| ODUkP-X-L/MT _A_Sk_MI _GAL_Enable[1…M] | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| ODUkP-X-L/MT_A_Sk Reporting |  |  |
| ODUkP-X-L/MT_A_Sk_MI_AcVcPT (see Table 15-8 of G.709) | 0..255 | -- |
| ODUkP-X-L/MT_A_Sk_MI_AcEXI (see Table 6-2 of G.7041) | 0..15 | -- |
| ODUkP-X-L/MT_A_Sk_MI_LastValidUPI (see Table 6-3 of G.7041) | 0..255 | -- |
| ETH/MT_A_So Provisioning |  |  |
| ETH/MT_A_So_MI_Label[1…M] | 16 to (2**20)-1 | Note-2 |
| ETH/MT_A_So_MI_LSPType[1…M] | E-LSP, L-LSP | Note-1 |
| ETH/MT_A_So_MI_CoS[1…M] | Note-1 | Note-1 |
| ETH/MT_A_So_PHB2TCMapping[1…M] | Note-1 | Note-1 |
| ETH/MT_A_So_MI_QoSEncodingMode[1…M] | A, B | Note-2 |
| ETH/MT_A_So_MI_Etype |  |  |
| ETH/MT_A_Sk Provisioning |  |  |
| ETH/MT_A_Sk_MI _LCK_Enable[1…M] | true, false | true |
| ETH/MT_A_Sk_MI _LCK_Period[1…M] | 1 s, 1 min | 1 s |
| ETH/MT_A_Sk_MI _LCK_CoS[1…M] | 0..7 | 7 |
| ETH/MT_A_Sk_MI _Admin_State | LCK, Normal | Normal |
| ETH/MT_A_Sk_MI _AIS_Enable[1…M] | true, false | true |
| ETH/MT_A_Sk_MI _AIS_Period[1…M] | 1 s, 1 min | 1 s |
| ETH/MT_A_Sk_MI _AIS_CoS[1…M] | 0..7 | 7 |

Note-1: According to [ITU-T G.8121/Y.1381]

Note-2: A value must be provided at provisioning.

# 3.2)	Updates to clause 8.6 “Diagnostic”

# Modify Table 8-3 as highlighted in the following replacement table for the deletions.

Table 8-3 G.8151/Y.1374  Provisioning and reporting for diagnostic trail termination function

| MI Signal | Value Range | Default Value |
| --- | --- | --- |
| MTDe_TT_So Provisioning |  |  |
| MTDe_TT_So_MI_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MTDe_TT_So_MI_TTLVALUE | 0..255 | 255 |
| MTDe_TT_So_MI_CV_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_So_MI_CV_Series() Note: The CV_Series parameters are OAM protocol-specific |  |  |
| MTDe_TT_So_MI_1TH_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_So_MI_1TH_Start(CoS, Length,Period) | CoS: 0..7 Length: 0..L. Note: The value of L depends on implementation, e.g. may be 2^32. Period: For further study | Default value of Length: 0 |
| MTDe_TT_So_MI_1TH_Terminate | -- | -- |
| MTDe_TT_So_MI_LMo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_So_MI_LMo_Start(CoS,Period) [1...MLMo] | CoS: 0..7 Period: 100ms, 1s, 10s |  |
| MTDe_TT_So_MI_LMo_Terminate[1...MLMo] | -- | -- |
| MTDe_TT_So_MI_DMo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_So_MI_DMo_Start(CoS,Test_ID,Length,Period)[1...MDMo] | CoS: 0..7 Test_ID: (Note 2) Length: 0..L. Note: The value of L depends on implementation, e.g. may be 2^32. Period: 1s, 10 s, 1 min | Default value of Length: 0 Default value of Period: 1 min |
| MTDe_TT_So_MI_DMo_Terminate[1...MDMo] | -- | -- |
| MTDe_TT_So_MI_1DMo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_So_MI_1DMo_Start(CoS,Test_ID,Length,Period)[1...M1DMo] | CoS: 0..7 Test_ID: (Note 2) Length: 0..L. Note: The value of L depends on implementation, e.g. may be 2^32. Period: 100ms, 1s, 10s | Default value of Length: 0 |
| MTDe_TT_So_MI_1DMo_Terminate[1...M1DMo] | -- | -- |
| MTDe_TT_So_MI_SLo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_So_MI_SLo_Start(CoS,Test_ID,Length,Period)[1...MSLo] | CoS: 0..7 Test_ID: (Note 2) Length: 0..L. Note: The value of L depends on implementation, e.g. may be 2^32. Period: 0.1 ms, 0.5 ms, 1 ms, 3.3 ms, 10 ms, 100 ms | Default value of Length: 0 Default value of Period: 10 ms |
| MTDe_TT_So_MI_SLo_Terminate[1...MSLo] | -- |  |
| MTDe_TT_So_MI_Admin_State | LCK, Normal | Normal |
| MTDe_TT_So_MI_Lock_Intsruct_Enable | true, false | true |
| MTDe_TT_So Reporting |  |  |
| MTDe_TT_So_MI_CV_Series_Result() Note: The CV_Series_Result parameters are OAM protocol-specific | -- | -- |
| MTDe_TT_So_MI_1TH_Result(Sent) | -- | -- |
| MTDe_TT_So_MI_LMo_Result(N_TF,N_LF,F_TF,F_LF)[1...MLMo] | -- | -- |
| MTDe_TT_So_MI_DMo_Result(count,B_FD[],F_FD[],N_FD[])[1...MDMo] | -- | -- |
| MTDe_TT_So_MI_SLo_Result(N_TF,N_LF,F_TF,F_LF)[1...MSLo] | -- | -- |
| MTDe_TT_Sk Provisioning |  |  |
| MTDe_TT_Sk_MI_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MTDe_TT_Sk_MI_CV_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_Sk_MI_1TH_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_Sk_MI_1TH_Start() |  |  |
| MTDe_TT_Sk_MI_1TH_Terminate | -- | -- |
| MTDe_TT_Sk_MI_LMo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_Sk_MI_DMo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_Sk_MI_1DMo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_Sk_MI_1DMo_Start(Test_ID)[1...M1DMo] | -- (Note 2) | -- |
| MTDe_TT_Sk_MI_1DMo_Terminate[1...M1DMo] | -- | -- |
| MTDe_TT_Sk_MI_SLo_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDe_TT_Sk Reporting |  |  |
| MTDe_TT_Sk_MI_1TH_Result(REC,CRC,BER,OO) | -- | -- |
| MTDe_TT_Sk_MI_1DMo_Result(count,N_FD[])[1...MDMo]] | -- | -- |
| MTDe_TT_Sk_MI_Admin_State_Request | Trigger to LCK, Trigger to Normal | -- |
| MTDi_TT_So Provisioning |  |  |
| MTDi_TT_So_MI_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MTDi_TT_So_MI_TTLVALUE | 0..255 | 255 |
| MTDe_TT_So_MI_MIP_ID | String; values are OAM protocol-specific | Note 1 |
| MTDi_TT_So_MI_CV_OAM_Tool | G.8113.1, G.8113.2 | N/A |
| MTDi_TT_Sk Provisioning |  |  |
| MTDi_TT_Sk_MI_GAL_Enable | true, false | Note: MI_GAL_Enable must be set to true on LSPs, to false on PWs using CW, and to true on Sections. Setting it to true on PWs not using CW is for further study. |
| MTDe_TT_Sk_MI_MIP_ID | String; values are OAM protocol-specific | Note 1 |
| MTDi_TT_Sk_MI_CV_OAM_Tool | G.8113.1, G.8113.2 | N/A |

Note-1: A value must be provided at provisioning.

Nore-2: The Test ID field is optional when this proactive measurement tool is used.

# 3.3)	Updates to clause 10.2 “Performance monitoring functions”

# Modify Table 10-1 as highlighted in the following replacement table for the addition.

| Table 10-1/G.8151/Y.1374 PM Management Information |  |
| --- | --- |
| PM Management Information | G.8121 Function |
| MT_TT_Sk_MI_pN_LF MT_TT_Sk_MI_pN_TF MT_TT_Sk_MI_pF_LF MT_TT_Sk_MI_pF_TF MT_TT_Sk_MI_pF_DS MT_TT_Sk_MI_pN_DS MT_TT_Sk_MI_pB_FD MT_TT_Sk_MI_pB_FDV MT_TT_Sk_MI_pN_FD MT_TT_Sk_MI_pN_FDV MT_TT_Sk_MI_pF_FD MT_TT_Sk_MI_pF_FDV | MT_TT_Sk |
| MT/ETH_A_Sk_MI_pFCSErrors | MT/ETH_A_Sk |

The EMF shall support the following functions:

- Notifying of the PM management information

__________________

____________________